Full Technical Code Explanation Report

Cobot_French.ino and Cobot_Terminal_French.py

Industrial-style technical documentation with source line references for constants, variables, functions, EEPROM logic, control systems, serial protocol, UI, and execution flow.

Source [Arduino]: Cobot_French.ino, 3024 lines.

Source [Python]: Cobot_Terminal_French.py, 1319 lines.

# 1. System Overview

The system is composed of an Arduino embedded controller and a Python desktop maintenance terminal. The Arduino source controls the physical robot: hardware pins, OLED feedback, buttons, encoder, LEDs, servo axes, inverse kinematics, gripper motor feedback, cycle execution, and EEPROM configuration. The Python source creates a French Tkinter interface that connects by USB serial, sends readable commands, parses status lines, simulates the Arduino when required, and gives the operator tabs for cycle testing, servo tests, button/LED tests, and position checks.

The responsibility boundary is important: Python does not directly control robot pins. It sends command lines; the Arduino validates state and executes physical actions. This keeps safety and hardware timing inside the embedded controller while giving a richer computer interface for maintenance.

# 2. Source Files Table

| Source | File | Role |
| --- | --- | --- |
| Source [Arduino] | Cobot_French.ino | Embedded hardware controller: pins, OLED, physical IHM, servos, gripper, kinematics, cycles, EEPROM, and serial/API handling. |
| Source [Python] | Cobot_Terminal_French.py | French desktop terminal: Tkinter GUI, serial connection, simulation mode, command sending, response parsing, dashboard update, and maintenance tabs. |

# 3. Libraries / Dependencies

| File | Library | Purpose | Source line |
| --- | --- | --- | --- |
| Arduino | Wire.h | I2C communication for OLED hardware. | Source [Arduino], line 1 |
| Arduino | Adafruit_GFX.h | Graphics/text primitives for OLED rendering. | Source [Arduino], line 2 |
| Arduino | Adafruit_SSD1306.h | SSD1306 OLED display driver. | Source [Arduino], line 3 |
| Arduino | Servo.h | PWM servo control for robot axes. | Source [Arduino], line 4 |
| Arduino | AutoPID.h | PID feedback controller support for the gripper. | Source [Arduino], line 5 |
| Arduino | MsTimer2.h | Timer service support for embedded timing. | Source [Arduino], line 6 |
| Arduino | EEPROM.h | Non-volatile persistent storage. | Source [Arduino], line 7 |
| Python | tkinter | GUI windowing and widgets. | Source [Python], line 1 |
| Python | tkinter | GUI windowing and widgets. | Source [Python], line 2 |
| Python | threading | Background serial reading and simulated cycle execution. | Source [Python], line 3 |
| Python | time | Connection delay and timing. | Source [Python], line 4 |
| Python | serial | USB serial communication via pyserial. | Source [Python], line 6 |
| Python | serial.tools.list_ports | Serial/Arduino port discovery. | Source [Python], line 7 |

# 4. Constants & Hardware Definitions (With Line Numbers)

| Variable | Type | Value | Meaning | Source |
| --- | --- | --- | --- | --- |
| PIN_SERVO_A | const int | 5 | Global value in PIN MAPPING. | Source [Arduino], line 12 |
| PIN_SERVO_B | const int | 9 | Global value in PIN MAPPING. | Source [Arduino], line 13 |
| PIN_SERVO_Z | const int | 11 | Global value in PIN MAPPING. | Source [Arduino], line 14 |
| PIN_GRIP_INA | const int | 6 | Global value in PIN MAPPING. | Source [Arduino], line 16 |
| PIN_GRIP_INB | const int | 7 | Global value in PIN MAPPING. | Source [Arduino], line 17 |
| PIN_GRIP_PWM | const int | 3 | Global value in PIN MAPPING. | Source [Arduino], line 18 |
| CURRENT_CLOSE_PIN | const int | A4 | Global value in PIN MAPPING. | Source [Arduino], line 20 |

| Variable | Type | Value | Meaning | Source |
| --- | --- | --- | --- | --- |
| CURRENT_OPEN_PIN | const int | A5 | Global value in PIN MAPPING. | Source [Arduino], line 21 |
| PIN_MAINTENANCE | const int | 44 | Global value in PIN MAPPING. | Source [Arduino], line 23 |
| BTN_PAUSE | const int | 18 | Global value in PIN MAPPING. | Source [Arduino], line 25 |
| BTN_START | const int | 22 | Global value in PIN MAPPING. | Source [Arduino], line 26 |
| BTN_INIT | const int | 24 | Global value in PIN MAPPING. | Source [Arduino], line 27 |
| ENC_CLK | const int | 40 | Global value in PIN MAPPING. | Source [Arduino], line 29 |
| ENC_DT | const int | 42 | Global value in PIN MAPPING. | Source [Arduino], line 30 |
| ENC_SW | const int | 46 | Global value in PIN MAPPING. | Source [Arduino], line 31 |
| LED_RUN | const int | 30 | Global value in PIN MAPPING. | Source [Arduino], line 33 |
| LED_PAUSE | const int | 32 | Global value in PIN MAPPING. | Source [Arduino], line 34 |
| LED_POWER | const int | 34 | Global value in PIN MAPPING. | Source [Arduino], line 35 |
| LED_INIT | const int | 36 | Global value in PIN MAPPING. | Source [Arduino], line 36 |
| SCREEN_WIDTH | #define | 128 | Preprocessor constant or macro used by the embedded controller. | Source [Arduino], line 41 |
| SCREEN_HEIGHT | #define | 64 | Preprocessor constant or macro used by the embedded controller. | Source [Arduino], line 42 |
| OLED_RESET | #define | -1 | Preprocessor constant or macro used by the embedded controller. | Source [Arduino], line 43 |
| SCREEN_ADDRESS | #define | 0x3C | Preprocessor constant or macro used by the embedded controller. | Source [Arduino], line 44 |
| servoA | Servo |  | Global value in ROBOT KINEMATICS. | Source [Arduino], line 51 |
| servoB | Servo |  | Global value in ROBOT KINEMATICS. | Source [Arduino], line 52 |
| servoZ | Servo |  | Global value in ROBOT KINEMATICS. | Source [Arduino], line 53 |
| L1 | float | 150.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 55 |
| L2 | float | 143.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 56 |
| offsetX | float | 20.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 57 |
| offsetZ | float | 75.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 58 |
| degOffsetA | float | -20.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 60 |
| degOffsetB | float | -10.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 61 |
| degOffsetZ | float | -2.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 62 |
| toolRadius | float | 40.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 64 |
| objectRadius | float | 32.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 65 |
| toolOffset | float | 65.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 66 |
| prehensionHeight | float | 50.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 67 |
| prehensionHeightBloc | float | 50.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 68 |
| toolHeight | float | -45 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 71 |
| maxStepA | float | 0.2 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 73 |
| maxStepB | float | 0.2 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 74 |
| maxStepZ | float | 0.5 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 75 |
| executionSpeedFactor | float | 1.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 77 |
| initX | float | 0.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 79 |
| initY | float | 130.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 80 |
| initZ | float | 120.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 81 |
| pauseReturnX | float | 0.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 83 |
| pauseReturnY | float | 130.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 84 |
| pauseReturnZ | float | 120.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 85 |
| pauseMoveInProgress | bool | false | Global value in ROBOT KINEMATICS. | Source [Arduino], line 87 |

| Variable | Type | Value | Meaning | Source |
| --- | --- | --- | --- | --- |
| currentX | float | 0.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 89 |
| currentY | float | 130.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 90 |
| currentZ | float | 120.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 91 |
| currentCmdA | float | 90.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 93 |
| currentCmdB | float | 90.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 94 |
| currentCmdZ | float | 90.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 95 |
| x | float |  | Global value in ROBOT KINEMATICS. | Source [Arduino], line 98 |
| y | float |  | Global value in ROBOT KINEMATICS. | Source [Arduino], line 99 |
| z | float |  | Global value in ROBOT KINEMATICS. | Source [Arduino], line 100 |
| a | float |  | Global value in ROBOT KINEMATICS. | Source [Arduino], line 104 |
| b | float |  | Global value in ROBOT KINEMATICS. | Source [Arduino], line 105 |
| z | float |  | Global value in ROBOT KINEMATICS. | Source [Arduino], line 106 |
| reachable | bool |  | Global value in ROBOT KINEMATICS. | Source [Arduino], line 107 |
| NB_CYCLES | const int | 2 | Global value in CYCLE POSITIONS. | Source [Arduino], line 113 |
| NB_OBJECTS | const int | 3 | Global value in CYCLE POSITIONS. | Source [Arduino], line 114 |
| NB_POSITIONS | const int | 4 | Global value in CYCLE POSITIONS. | Source [Arduino], line 115 |
| OBJ_GOMME | const int | 0 | Global value in CYCLE POSITIONS. | Source [Arduino], line 117 |
| OBJ_GOBELET | const int | 1 | Global value in CYCLE POSITIONS. | Source [Arduino], line 118 |
| OBJ_CYLINDRE | const int | 2 | Global value in CYCLE POSITIONS. | Source [Arduino], line 119 |
| POS_convoyeurEntree | const int | 0 | Global value in CYCLE POSITIONS. | Source [Arduino], line 121 |
| POS_machineA | const int | 1 | Global value in CYCLE POSITIONS. | Source [Arduino], line 122 |
| POS_machineB | const int | 2 | Global value in CYCLE POSITIONS. | Source [Arduino], line 123 |
| POS_convoyeurSortie | const int | 3 | Global value in CYCLE POSITIONS. | Source [Arduino], line 124 |
| bootIgnoreButtonsMs | const unsigned long | 1500 | Global value in HMI GLOBAL STATES. | Source [Arduino], line 208 |
| debounceDelay | const unsigned long | 50 | Global value in HMI GLOBAL STATES. | Source [Arduino], line 212 |
| encoderMoveDelay | const unsigned long | 80 | Global value in HMI GLOBAL STATES. | Source [Arduino], line 217 |
| encoderButtonDelay | const unsigned long | 300 | Global value in HMI GLOBAL STATES. | Source [Arduino], line 218 |
| statusRefreshDelay | const unsigned long | 500 | Global value in HMI GLOBAL STATES. | Source [Arduino], line 221 |
| homeMenu | const char*[] | {"Init", "Config", "Reglages", "Info", "Statut"} | Global value in HMI GLOBAL STATES. | Source [Arduino], line 223 |
| initMenu | const char*[] | {"Repos + Init", "Retour"} | Global value in HMI GLOBAL STATES. | Source [Arduino], line 224 |
| configMenu | const char*[] | {"Choix Cycle", "Choix Objet", "Choix Mode", "Nb Pieces", "Retour"} | Global value in HMI GLOBAL STATES. | Source [Arduino], line 225 |
| cycleMenu | const char*[] | {"Cycle 1", "Cycle 2", "Retour"} | Global value in HMI GLOBAL STATES. | Source [Arduino], line 226 |
| objectMenu | const char*[] | {"Gomme", "Gobelet", "Cylindre", "Retour"} | Global value in HMI GLOBAL STATES. | Source [Arduino], line 227 |
| modeMenu | const char*[] | {"ECO", "Normal", "RAPIDE", "Retour"} | Global value in HMI GLOBAL STATES. | Source [Arduino], line 228 |
| combineMenu | const char*[] | {"1", "2", "3", "4", "5", "Retour"} | Global value in HMI GLOBAL STATES. | Source [Arduino], line 229 |
| EEPROM_MAGIC | const unsigned long | 20260427 | Global value in EEPROM SETTINGS. | Source [Arduino], line 265 |
| EEPROM_ADDRESS | const int | 0 | Global value in EEPROM SETTINGS. | Source [Arduino], line 266 |
| KP_GRIP | #define | 90.0 | Preprocessor constant or macro used by the embedded controller. | Source [Arduino], line 477 |
| KI_GRIP | #define | 0.0 | Preprocessor constant or macro used by the embedded controller. | Source [Arduino], line 478 |
| KD_GRIP | #define | 2.5 | Preprocessor constant or macro used by the embedded controller. | Source [Arduino], line 479 |
| Variable | Type | Value | Meaning | Source |
| alphaGrip | const double | 0.25 | Global value in GRIPPER PID. | Source [Arduino], line 494 |
| maxGripPWMVariation | const double | 15.0 | Global value in GRIPPER PID. | Source [Arduino], line 497 |
| USE_REAL_ARDUINO | Python constant/list/dict | True | Top-level configuration/state/widget variable in CONNECTION SETTINGS. | Source [Python], line 12 |
| SERIAL_PORT | Python constant/list/dict | "COM6" | Top-level configuration/state/widget variable in CONNECTION SETTINGS. | Source [Python], line 13 |
| BAUD_RATE | Python constant/list/dict | 9600 | Top-level configuration/state/widget variable in CONNECTION SETTINGS. | Source [Python], line 14 |
| objects | Python constant/list/dict | ["Gomme", "Gobelet", "Cylindre"] | Top-level configuration/state/widget variable in CONSTANTS. | Source [Python], line 19 |
| modes | Python constant/list/dict | ["ÉCO", "Normal", "RAPIDE"] | Top-level configuration/state/widget variable in CONSTANTS. | Source [Python], line 20 |
| cycle_names | Python constant/list/dict | { 0: "Cycle 1", 1: "Cycle 2", } | Top-level configuration/state/widget variable in CONSTANTS. | Source [Python], line 22 |
| position_names | Python constant/list/dict | [ "Entrée", "Machine A", "Machine B", "Sortie", ] | Top-level configuration/state/widget variable in Positions available for position testing in Tab 4.. | Source [Python], line 31 |
| position_code_names | Python constant/list/dict | [ "POS_convoyeurEntree", "POS_machineA", "POS_machineB", "POS_convoyeurSortie", ] | Top-level configuration/state/widget variable in Positions available for position testing in Tab 4.. | Source [Python], line 38 |
| position_code_names | Python constant/list/dict | [ "POS_convoyeurEntree", "POS_machineA", "POS_machineB", "POS_convoyeurSortie", ] | Top-level configuration/state/widget variable in Positions available for position testing in Tab 4.. | Source [Python], line 45 |

# 5. Global Variables & Data Structures (With Line Numbers)

## 5.1 Arduino global variables

| Variable | Type | Value | Meaning | Section | Source |
| --- | --- | --- | --- | --- | --- |
| screenState | ScreenState | SCREEN_HOME | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 181 |
| isInitialized | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 183 |
| isInitializing | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 184 |
| isRunning | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 185 |
| isPaused | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 186 |
| pauseInterruptRequest | volatile bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 188 |
| terminalModeActive | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 190 |
| manualServoTestDirty | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 191 |
| terminalButtonTestMode | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 192 |
| terminalLedTestMode | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 193 |
| ledTestInit | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 194 |
| ledTestRun | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 195 |
| ledTestPause | bool | false | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 196 |
| ledTestPower | bool | true | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 197 |
| lastTerminalContact | unsigned long | 0 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 198 |
| menuIndex | int | 0 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 200 |
| currentCycleType | int | 0 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 201 |
| currentObject | int | 0 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 202 |
| currentMode | int | 1 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 203 |
| currentCombine | int | 0 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 204 |
| currentPiece | int | 0 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 205 |
| bootTime | unsigned long | 0 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 207 |

| Variable | Type | Value | Meaning | Section | Source |
| --- | --- | --- | --- | --- | --- |
| previousButtonState | bool[3] | {HIGH, HIGH, HIGH} | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 210 |
| lastDebounceTime | unsigned long[3] | {0, 0, 0} | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 211 |
| lastEncCLK | int | HIGH | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 214 |
| lastEncoderMoveTime | unsigned long | 0 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 215 |
| lastEncoderButtonTime | unsigned long | 0 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 216 |
| lastStatusUpdate | unsigned long | 0 | Global value in HMI GLOBAL STATES. | HMI GLOBAL STATES | Source [Arduino], line 220 |
| consigneOuverture | double | 1.80 | Global value in IMPORTANT: declared here before EEPROM functions use them. | IMPORTANT: declared here before EEPROM functions use them | Source [Arduino], line 235 |
| consigneFermeture | double | 1.40 | Global value in IMPORTANT: declared here before EEPROM functions use them. | IMPORTANT: declared here before EEPROM functions use them | Source [Arduino], line 236 |
| seuilOuvertureMax | double | 2.30 | Global value in IMPORTANT: declared here before EEPROM functions use them. | IMPORTANT: declared here before EEPROM functions use them | Source [Arduino], line 238 |
| seuilFermetureMax | double | 2.60 | Global value in IMPORTANT: declared here before EEPROM functions use them. | IMPORTANT: declared here before EEPROM functions use them | Source [Arduino], line 239 |
| dx | float |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 245 |
| dy | float |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 246 |
| dz | float |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 247 |
| openSetpoint | float |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 251 |
| closeSetpoint | float |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 252 |
| magic | unsigned long |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 256 |
| offsets | PositionOffset[NB_CYCLES][ NB_OBJECTS][NB_POSITIO NS] |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 257 |
| grip | GripSettings[NB_OBJECTS] |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 258 |
| savedSpeedFactor | float |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 259 |
| savedServoOffsetA | float |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 260 |
| savedServoOffsetB | float |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 261 |
| savedServoOffsetZ | float |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 262 |
| eepromData | EEPROMData |  | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 268 |
| maintCycle | int | 0 | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 270 |
| maintObject | int | 0 | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 271 |
| maintPosition | int | 0 | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 272 |
| offsetStep | float | 1.0 | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 274 |
| gripStep | float | 0.05 | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 275 |
| speedStep | float | 0.1 | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 276 |
| servoOffsetStep | float | 1.0 | Global value in EEPROM SETTINGS. | EEPROM SETTINGS | Source [Arduino], line 277 |
| gripperState | GripperState | GRIPPER_STOPPED | Global value in GRIPPER PID. | GRIPPER PID | Source [Arduino], line 487 |
| gripperPWM_PID | double | 0.0 | Global value in GRIPPER PID. | GRIPPER PID | Source [Arduino], line 489 |
| measuredTension | double | 0.0 | Global value in GRIPPER PID. | GRIPPER PID | Source [Arduino], line 490 |
| gripperSetpoint | double | 0.0 | Global value in GRIPPER PID. | GRIPPER PID | Source [Arduino], line 491 |
| filteredTension | double | 0.0 | Global value in GRIPPER PID. | GRIPPER PID | Source [Arduino], line 493 |
| previousGripPWM | double | 0.0 | Global value in GRIPPER PID. | GRIPPER PID | Source [Arduino], line 496 |
| gripperDone | bool | false | Global value in GRIPPER PID. | GRIPPER PID | Source [Arduino], line 499 |
| objectDetected | bool | false | Global value in GRIPPER PID. | GRIPPER PID | Source [Arduino], line 500 |
| openLimitDetected | bool | false | Global value in GRIPPER PID. | GRIPPER PID | Source [Arduino], line 501 |
| Variable | Type | Value | Meaning | Section | Source |
| tick10ms | volatile bool | false | Global value in INTERRUPTS. | INTERRUPTS | Source [Arduino], line 540 |
| maintConsoleState | MaintenanceConsoleState | MAINT_MAIN | Global value in MAINTENANCE MODE. | MAINTENANCE MODE | Source [Arduino], line 1631 |
| apiBuffer | String | "" | Global value in position: 0=POS_convoyeurEntree, 1=POS_machineA, 2=POS_machineB, 3=POS. | position: 0=POS_convoyeurEntree, 1=POS_machineA, 2=POS_machineB, 3=POS | Source [Arduino], line 2488 |

## 5.2 Arduino data structures and fields

| Structure / Field | Type | Meaning | Group | Source |
| --- | --- | --- | --- | --- |
| Point3D | struct | Declared data type used by the controller. | ROBOT KINEMATICS | Source [Arduino], line 97 |
| JointCmd | struct | Declared data type used by the controller. | ROBOT KINEMATICS | Source [Arduino], line 103 |
| ScreenState | enum | Declared data type used by the controller. | HMI GLOBAL STATES | Source [Arduino], line 167 |
| PositionOffset | struct | Declared data type used by the controller. | EEPROM SETTINGS | Source [Arduino], line 244 |
| GripSettings | struct | Declared data type used by the controller. | EEPROM SETTINGS | Source [Arduino], line 250 |
| EEPROMData | struct | Declared data type used by the controller. | EEPROM SETTINGS | Source [Arduino], line 255 |
| GripperState | enum | Declared data type used by the controller. | GRIPPER PID | Source [Arduino], line 481 |
| MaintenanceConsoleState | enum | Declared data type used by the controller. | MAINTENANCE MODE | Source [Arduino], line 1622 |
| Point3D.x | float | Field of Point3D. | Point3D | Source [Arduino], line 98 |
| Point3D.y | float | Field of Point3D. | Point3D | Source [Arduino], line 99 |
| Point3D.z | float | Field of Point3D. | Point3D | Source [Arduino], line 100 |
| JointCmd.a | float | Field of JointCmd. | JointCmd | Source [Arduino], line 104 |
| JointCmd.b | float | Field of JointCmd. | JointCmd | Source [Arduino], line 105 |
| JointCmd.z | float | Field of JointCmd. | JointCmd | Source [Arduino], line 106 |
| JointCmd.reachable | bool | Field of JointCmd. | JointCmd | Source [Arduino], line 107 |
| PositionOffset.dx | float | Field of PositionOffset. | PositionOffset | Source [Arduino], line 245 |
| PositionOffset.dy | float | Field of PositionOffset. | PositionOffset | Source [Arduino], line 246 |
| PositionOffset.dz | float | Field of PositionOffset. | PositionOffset | Source [Arduino], line 247 |
| GripSettings.openSetpoint | float | Field of GripSettings. | GripSettings | Source [Arduino], line 251 |
| GripSettings.closeSetpoint | float | Field of GripSettings. | GripSettings | Source [Arduino], line 252 |
| EEPROMData.magic | unsigned long | Field of EEPROMData. | EEPROMData | Source [Arduino], line 256 |
| EEPROMData.offsets | PositionOffset[NB_CYCLES][NB_O BJECTS][NB_POSITIONS] | Field of EEPROMData. | EEPROMData | Source [Arduino], line 257 |
| EEPROMData.grip | GripSettings[NB_OBJECTS] | Field of EEPROMData. | EEPROMData | Source [Arduino], line 258 |
| EEPROMData.savedSpeedFactor | float | Field of EEPROMData. | EEPROMData | Source [Arduino], line 259 |
| EEPROMData.savedServoOffsetA | float | Field of EEPROMData. | EEPROMData | Source [Arduino], line 260 |
| EEPROMData.savedServoOffsetB | float | Field of EEPROMData. | EEPROMData | Source [Arduino], line 261 |
| EEPROMData.savedServoOffsetZ | float | Field of EEPROMData. | EEPROMData | Source [Arduino], line 262 |

## 5.3 Python top-level variables, state, and GUI objects

| Variable | Type | Value | Meaning | Section | Source |
| --- | --- | --- | --- | --- | --- |
| USE_REAL_ARDUINO | Python top-level assignment | True | Top-level configuration/state/widget variable in CONNECTION SETTINGS. | CONNECTION SETTINGS | Source [Python], line 12 |
| SERIAL_PORT | Python top-level assignment | "COM6" | Top-level configuration/state/widget variable in CONNECTION SETTINGS. | CONNECTION SETTINGS | Source [Python], line 13 |

| Variable | Type | Value | Meaning | Section | Source |
| --- | --- | --- | --- | --- | --- |
| BAUD_RATE | Python top-level assignment | 9600 | Top-level configuration/state/widget variable in CONNECTION SETTINGS. | CONNECTION SETTINGS | Source [Python], line 14 |
| objects | Python top-level assignment | ["Gomme", "Gobelet", "Cylindre"] | Top-level configuration/state/widget variable in CONSTANTS. | CONSTANTS | Source [Python], line 19 |
| modes | Python top-level assignment | ["ÉCO", "Normal", "RAPIDE"] | Top-level configuration/state/widget variable in CONSTANTS. | CONSTANTS | Source [Python], line 20 |
| cycle_names | Python top-level assignment | { 0: "Cycle 1", 1: "Cycle 2", } | Top-level configuration/state/widget variable in CONSTANTS. | CONSTANTS | Source [Python], line 22 |
| position_names | Python top-level assignment | [ "Entrée", "Machine A", "Machine B", "Sortie", ] | Top-level configuration/state/widget variable in Positions available for position testing in Tab 4.. | Positions available for position testing in Tab 4. | Source [Python], line 31 |
| position_code_names | Python top-level assignment | [ "POS_convoyeurEntree", "POS_machineA", "POS_machineB", "POS_convoyeurSortie", ] | Top-level configuration/state/widget variable in Positions available for position testing in Tab 4.. | Positions available for position testing in Tab 4. | Source [Python], line 38 |
| position_code_names | Python top-level assignment | [ "POS_convoyeurEntree", "POS_machineA", "POS_machineB", "POS_convoyeurSortie", ] | Top-level configuration/state/widget variable in Positions available for position testing in Tab 4.. | Positions available for position testing in Tab 4. | Source [Python], line 45 |
| arduino | Python top-level assignment | None | Top-level configuration/state/widget variable in GLOBAL STATE. | GLOBAL STATE | Source [Python], line 55 |
| reader_running | Python top-level assignment | False | Top-level configuration/state/widget variable in GLOBAL STATE. | GLOBAL STATE | Source [Python], line 56 |
| state | Python top-level assignment | { "connection": "DÉCONNECTÉ", "robot_state": "NOT_INIT", "init": 0, "initializing": | Top-level configuration/state/widget variable in GLOBAL STATE. | GLOBAL STATE | Source [Python], line 58 |
| root | Python top-level assignment | tk.Tk() | Top-level configuration/state/widget variable in WINDOW. | WINDOW | Source [Python], line 838 |
| connection_var | Python top-level assignment | tk.StringVar(value="DÉCONNECTÉ") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 846 |
| robot_state_var | Python top-level assignment | tk.StringVar(value="NOT_INIT") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 847 |
| init_var | Python top-level assignment | tk.StringVar(value="0") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 849 |
| run_var | Python top-level assignment | tk.StringVar(value="0") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 850 |
| pause_var | Python top-level assignment | tk.StringVar(value="0") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 851 |
| cycle_var | Python top-level assignment | tk.StringVar(value="Cycle 1") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 853 |
| object_var | Python top-level assignment | tk.StringVar(value="Gomme") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 854 |
| mode_var | Python top-level assignment | tk.StringVar(value="Normal") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 855 |
| pieces_var | Python top-level assignment | tk.StringVar(value="1") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 856 |
| current_piece_var | Python top-level assignment | tk.StringVar(value="0") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 857 |
| speed_var | Python top-level assignment | tk.StringVar(value="1.00") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 858 |
| current_x_var | Python top-level assignment | tk.StringVar(value="?") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 860 |
| current_y_var | Python top-level assignment | tk.StringVar(value="?") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 861 |
| current_z_var | Python top-level assignment | tk.StringVar(value="?") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 862 |
| last_position_var | Python top-level assignment | tk.StringVar(value="None") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 863 |
| servo_a_live_var | Python top-level assignment | tk.StringVar(value="90.0") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 864 |
| servo_b_live_var | Python top-level assignment | tk.StringVar(value="90.0") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 865 |
| servo_z_live_var | Python top-level assignment | tk.StringVar(value="90.0") | Top-level configuration/state/widget variable in VARIABLES. | VARIABLES | Source [Python], line 866 |
| dash | Python top-level assignment | tk.LabelFrame(root, text="État en temps réel", padx=10, pady=10) | Top-level configuration/state/widget variable in DASHBOARD. | DASHBOARD | Source [Python], line 880 |
| main | Python top-level assignment | tk.Frame(root) | Top-level configuration/state/widget variable in MAIN AREA WITH TABS. | MAIN AREA WITH TABS | Source [Python], line 919 |
| notebook | Python top-level assignment | ttk.Notebook(main) | Top-level configuration/state/widget variable in MAIN AREA WITH TABS. | MAIN AREA WITH TABS | Source [Python], line 922 |
| config_tab | Python top-level assignment | tk.Frame(notebook) | Top-level configuration/state/widget variable in TAB 1 - CONFIGURATION. | TAB 1 - CONFIGURATION | Source [Python], line 929 |
| config_box | Python top-level assignment | tk.LabelFrame(config_tab, text="Configuration du test de cycle", padx=20, pady=20) | Top-level configuration/state/widget variable in TAB 1 - CONFIGURATION. | TAB 1 - CONFIGURATION | Source [Python], line 932 |

| Variable | Type | Value | Meaning | Section | Source |
| --- | --- | --- | --- | --- | --- |
| config_cycle | Python top-level assignment | tk.IntVar(value=1) | Top-level configuration/state/widget variable in TAB 1 - CONFIGURATION. | TAB 1 - CONFIGURATION | Source [Python], line 936 |
| config_object | Python top-level assignment | tk.StringVar(value="Gomme") | Top-level configuration/state/widget variable in TAB 1 - CONFIGURATION. | TAB 1 - CONFIGURATION | Source [Python], line 940 |
| config_mode | Python top-level assignment | tk.StringVar(value="Normal") | Top-level configuration/state/widget variable in TAB 1 - CONFIGURATION. | TAB 1 - CONFIGURATION | Source [Python], line 944 |
| config_pieces | Python top-level assignment | tk.IntVar(value=1) | Top-level configuration/state/widget variable in TAB 1 - CONFIGURATION. | TAB 1 - CONFIGURATION | Source [Python], line 948 |
| config_speed | Python top-level assignment | tk.DoubleVar(value=1.00) | Top-level configuration/state/widget variable in TAB 1 - CONFIGURATION. | TAB 1 - CONFIGURATION | Source [Python], line 952 |
| oled_box | Python top-level assignment | tk.LabelFrame(config_box, text="Aperçu OLED simulé", padx=15, pady=15) | Top-level configuration/state/widget variable in Fake OLED is only shown in Tab 1 because this tab is the cycle/config . | Fake OLED is only shown in Tab 1 because this tab is the cycle/config | Source [Python], line 964 |
| oled_canvas | Python top-level assignment | tk.Canvas( oled_box, width=300, height=170, bg="black", highlightthickness=2, | Top-level configuration/state/widget variable in Fake OLED is only shown in Tab 1 because this tab is the cycle/config . | Fake OLED is only shown in Tab 1 because this tab is the cycle/config | Source [Python], line 967 |
| ihm_box | Python top-level assignment | tk.LabelFrame(config_box, text="Boutons de commande cycle", padx=16, pady=16) | Top-level configuration/state/widget variable in IHM cycle control buttons are only shown in Tab 1 because this tab is . | IHM cycle control buttons are only shown in Tab 1 because this tab is | Source [Python], line 978 |
| servo_tab | Python top-level assignment | tk.Frame(notebook) | Top-level configuration/state/widget variable in TAB 2 - SINGLE SERVO TESTS. | TAB 2 - SINGLE SERVO TESTS | Source [Python], line 1022 |
| servo_box | Python top-level assignment | tk.LabelFrame(servo_tab, text="Tests individuels des servos", padx=20, pady=20) | Top-level configuration/state/widget variable in TAB 2 - SINGLE SERVO TESTS. | TAB 2 - SINGLE SERVO TESTS | Source [Python], line 1025 |
| servo_a_var | Python top-level assignment | add_servo_control(1, "A", "Servo A") | Top-level configuration/state/widget variable in TAB 2 - SINGLE SERVO TESTS. | TAB 2 - SINGLE SERVO TESTS | Source [Python], line 1067 |
| servo_b_var | Python top-level assignment | add_servo_control(2, "B", "Servo B") | Top-level configuration/state/widget variable in TAB 2 - SINGLE SERVO TESTS. | TAB 2 - SINGLE SERVO TESTS | Source [Python], line 1068 |
| servo_z_var | Python top-level assignment | add_servo_control(3, "Z", "Servo Z") | Top-level configuration/state/widget variable in TAB 2 - SINGLE SERVO TESTS. | TAB 2 - SINGLE SERVO TESTS | Source [Python], line 1069 |
| button_led_tab | Python top-level assignment | tk.Frame(notebook) | Top-level configuration/state/widget variable in TAB 3 - BUTTON AND LED TESTS. | TAB 3 - BUTTON AND LED TESTS | Source [Python], line 1082 |
| button_led_box | Python top-level assignment | tk.LabelFrame(button_led_tab, text="Tests boutons et LED", padx=20, pady=20) | Top-level configuration/state/widget variable in TAB 3 - BUTTON AND LED TESTS. | TAB 3 - BUTTON AND LED TESTS | Source [Python], line 1085 |
| button_test_status_var | Python top-level assignment | tk.StringVar(value="Test boutons inactif.") | Top-level configuration/state/widget variable in TAB 3 - BUTTON AND LED TESTS. | TAB 3 - BUTTON AND LED TESTS | Source [Python], line 1096 |
| position_tab | Python top-level assignment | tk.Frame(notebook) | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1143 |
| position_box | Python top-level assignment | tk.LabelFrame(position_tab, text="Tests de position", padx=16, pady=16) | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1146 |
| position_cycle_var | Python top-level assignment | tk.IntVar(value=1) | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1160 |
| position_object_var | Python top-level assignment | tk.StringVar(value=objects[0]) | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1161 |
| position_name_var | Python top-level assignment | tk.StringVar(value=position_names[0]) | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1162 |
| position_command_var | Python top-level assignment | tk.StringVar(value="GOTO_POSITIO N 0 0 0") | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1163 |
| position_status_var | Python top-level assignment | tk.StringVar(value="Sélectionner un cycle, un objet et une position.") | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1164 |
| Variable | Type | Value | Meaning | Section | Source |
| left_panel | Python top-level assignment | tk.Frame(position_box) | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1170 |
| right_panel | Python top-level assignment | tk.LabelFrame(position_box, text="Position retournée par Arduino", padx=12, pady=10) | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1173 |
| quick_box | Python top-level assignment | tk.LabelFrame(left_panel, text="Boutons rapides de position", padx=10, pady=10) | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1220 |
| button_row | Python top-level assignment | tk.Frame(left_panel) | Top-level configuration/state/widget variable in TAB 4 - POSITION CHECK. | TAB 4 - POSITION CHECK | Source [Python], line 1239 |
| log_frame | Python top-level assignment | tk.LabelFrame(root, text="Journal série", padx=5, pady=5) | Top-level configuration/state/widget variable in LOG. | LOG | Source [Python], line 1294 |
| log_box | Python top-level assignment | tk.Text(log_frame, height=9) | Top-level configuration/state/widget variable in LOG. | LOG | Source [Python], line 1297 |

## 5.4 Local variables / local assignments by function

| Function | Variable | Type | Value | Meaning | Source |
| --- | --- | --- | --- | --- | --- |
| applyMachinePieceCentering | ux | float | target.x / r | Local variable used inside applyMachinePieceCentering. | Source [Arduino], line 426 |
| applyMachinePieceCentering | uy | float | target.y / r | Local variable used inside applyMachinePieceCentering. | Source [Arduino], line 427 |
| applyMachinePieceCentering | pieceCenterCorrection | float | toolOffset - toolLength | Local variable used inside applyMachinePieceCentering. | Source [Arduino], line 429 |
| getCorrectedPosition | base | Point3D | basePositions[cycle][position] | Local variable used inside getCorrectedPosition. | Source [Arduino], line 436 |
| getCorrectedPosition | corrected | Point3D |  | Local variable used inside getCorrectedPosition. | Source [Arduino], line 439 |
| getCorrectedPrehensionPositio n | base | Point3D | basePositions[cycle][position] | Local variable used inside getCorrectedPrehensionPosition. | Source [Arduino], line 450 |
| getCorrectedPrehensionPositio n | corrected | Point3D |  | Local variable used inside getCorrectedPrehensionPosition. | Source [Arduino], line 453 |
| buttonPressed | pressed | bool | false | Local variable used inside buttonPressed. | Source [Arduino], line 558 |
| drawMenu | startIdx | int | 0 | Local variable used inside drawMenu. | Source [Arduino], line 619 |
| refreshScreen | break | default: |  | Local variable used inside refreshScreen. | Source [Arduino], line 713 |
| executeCurrentScreen | screenState | else | SCREEN_HOME | Local variable used inside executeCurrentScreen. | Source [Arduino], line 851 |
| updateGripperPID | rawTension | double |  | Local variable used inside updateGripperPID. | Source [Arduino], line 964 |
| limitStep | delta | float | targetValue - currentValue | Local variable used inside limitStep. | Source [Arduino], line 1099 |
| computeIK | out | JointCmd |  | Local variable used inside computeIK. | Source [Arduino], line 1108 |
| computeIK | rWrist | float | rTool - toolLength | Local variable used inside computeIK. | Source [Arduino], line 1114 |
| computeIK | zWrist | float | z - toolHeight | Local variable used inside computeIK. | Source [Arduino], line 1115 |
| computeIK | rp | float | rWrist - offsetX | Local variable used inside computeIK. | Source [Arduino], line 1117 |
| computeIK | zp | float | zWrist - offsetZ | Local variable used inside computeIK. | Source [Arduino], line 1118 |
| computeIK | cmdA | float | degA + degOffsetA | Local variable used inside computeIK. | Source [Arduino], line 1131 |
| computeIK | cmdB | float | degB + degOffsetB | Local variable used inside computeIK. | Source [Arduino], line 1132 |
| computeIK | cmdZ | float | degZ + degOffsetZ | Local variable used inside computeIK. | Source [Arduino], line 1133 |
| moveJointsSmoothTo | done | bool | false | Local variable used inside moveJointsSmoothTo. | Source [Arduino], line 1153 |
| trapezoidalProfile | s | float | 0.0 | Local variable used inside trapezoidalProfile. | Source [Arduino], line 1179 |
| trapezoidalProfile | du | float | 1.0 - u | Local variable used inside trapezoidalProfile. | Source [Arduino], line 1188 |
| moveRectangular | pStart | Point3D | {currentX, currentY, currentZ} | Local variable used inside moveRectangular. | Source [Arduino], line 1238 |
| moveRectangular | pUp | Point3D | {currentX, currentY, zLift} | Local variable used inside moveRectangular. | Source [Arduino], line 1242 |
| moveRectangular | pMid | Point3D | {xf, yf, zLift} | Local variable used inside moveRectangular. | Source [Arduino], line 1243 |
| moveRectangular | pEnd | Point3D | {xf, yf, zf} | Local variable used inside moveRectangular. | Source [Arduino], line 1244 |

| Function | Variable | Type | Value | Meaning | Source |
| --- | --- | --- | --- | --- | --- |
| printSelectedMaintenanceTarg et | off | PositionOffset | eepromData.offsets[maintCycle][mai ntObject][maintPosition] | Local variable used inside printSelectedMaintenanceTarget. | Source [Arduino], line 1410 |
| printSelectedMaintenanceTarg et | base | Point3D | basePositions[maintCycle][maintPos ition] | Local variable used inside printSelectedMaintenanceTarget. | Source [Arduino], line 1411 |
| printPositionTable | base | Point3D | basePositions[cycle][p] | Local variable used inside printPositionTable. | Source [Arduino], line 1443 |
| printPositionTable | off | PositionOffset | eepromData.offsets[cycle][object][p] | Local variable used inside printPositionTable. | Source [Arduino], line 1444 |
| updateSelectedOffset | off | PositionOffset* | &eepromData.offsets[maintCycle][m aintObject][maintPosition] | Local variable used inside updateSelectedOffset. | Source [Arduino], line 1529 |
| readSerialCommand | cmd | String | "" | Local variable used inside readSerialCommand. | Source [Arduino], line 1676 |
| ihmInitializeAction | initOK | bool | true | Local variable used inside ihmInitializeAction. | Source [Arduino], line 1952 |
| setLedByName | pin | int | -1 | Local variable used inside setLedByName. | Source [Arduino], line 2078 |
| pulseLedByName | pin | int | -1 | Local variable used inside pulseLedByName. | Source [Arduino], line 2107 |
| pulseLedByName | previousLedTestMode | bool | terminalLedTestMode | Local variable used inside pulseLedByName. | Source [Arduino], line 2119 |
| getCycleTimes | speedMult | float | 1.0 | Local variable used inside getCycleTimes. | Source [Arduino], line 2163 |
| getCycleTimes | tUpBase | unsigned long | 500 | Local variable used inside getCycleTimes. | Source [Arduino], line 2171 |
| getCycleTimes | tXYBase | unsigned long | 500 | Local variable used inside getCycleTimes. | Source [Arduino], line 2172 |
| getCycleTimes | tDownBase | unsigned long | 500 | Local variable used inside getCycleTimes. | Source [Arduino], line 2173 |
| SequenceCycle1ForObject | tUp | unsigned long |  | Local variable used inside SequenceCycle1ForObject. | Source [Arduino], line 2238 |
| SequenceCycle1ForObject | tXY | unsigned long |  | Local variable used inside SequenceCycle1ForObject. | Source [Arduino], line 2239 |
| SequenceCycle1ForObject | tDown | unsigned long |  | Local variable used inside SequenceCycle1ForObject. | Source [Arduino], line 2240 |
| SequenceCycle2ForObject | tUp | unsigned long |  | Local variable used inside SequenceCycle2ForObject. | Source [Arduino], line 2276 |
| SequenceCycle2ForObject | tXY | unsigned long |  | Local variable used inside SequenceCycle2ForObject. | Source [Arduino], line 2277 |
| SequenceCycle2ForObject | tDown | unsigned long |  | Local variable used inside SequenceCycle2ForObject. | Source [Arduino], line 2278 |
| handleApiCommand | upper | String | cmd | Local variable used inside handleApiCommand. | Source [Arduino], line 2653 |
| handleApiCommand | newCombine | int | pieces - 1 | Local variable used inside handleApiCommand. | Source [Arduino], line 2707 |
| find_arduino_port | ports | Python local assignment | serial.tools.list_ports.comports() | Local value used inside find_arduino_port. | Source [Python], line 94 |
| find_arduino_port | desc | Python local assignment | port.description.lower() | Local value used inside find_arduino_port. | Source [Python], line 97 |
| find_arduino_port | device | Python local assignment | port.device | Local value used inside find_arduino_port. | Source [Python], line 98 |
| connect_arduino | port | Python local assignment | resolve_serial_port() | Local value used inside connect_arduino. | Source [Python], line 128 |
| connect_arduino | arduino | Python local assignment | serial.Serial(port, BAUD_RATE, timeout=0.1) | Local value used inside connect_arduino. | Source [Python], line 133 |
| connect_arduino | response | Python local assignment | arduino.readline().decode(errors="ig nore").strip() | Local value used inside connect_arduino. | Source [Python], line 146 |
| connect_arduino | reader_running | Python local assignment | True | Local value used inside connect_arduino. | Source [Python], line 156 |
| disconnect_arduino | reader_running | Python local assignment | False | Local value used inside disconnect_arduino. | Source [Python], line 173 |
| disconnect_arduino | arduino | Python local assignment | None | Local value used inside disconnect_arduino. | Source [Python], line 181 |
| read_arduino | raw | Python local assignment | arduino.readline().decode(errors="ig nore").strip() | Local value used inside read_arduino. | Source [Python], line 215 |
| simulate_response | parts | Python local assignment | cmd.strip().split() | Local value used inside simulate_response. | Source [Python], line 227 |
| simulate_response | c0 | Python local assignment | parts[0].upper() | Local value used inside simulate_response. | Source [Python], line 231 |
| simulate_response | cycle_index | Python local assignment | int(parts[1]) | Local value used inside simulate_response. | Source [Python], line 351 |
| simulate_response | object_index | Python local assignment | int(parts[2]) | Local value used inside simulate_response. | Source [Python], line 352 |
| simulate_response | position_index | Python local assignment | int(parts[3]) | Local value used inside simulate_response. | Source [Python], line 353 |
| simulate_response | position_name | Python local assignment | position_names[position_index] if 0 <= position_index < len(position_names) else | Local value used inside simulate_response. | Source [Python], line 360 |

| Function | Variable | Type | Value | Meaning | Source |
| --- | --- | --- | --- | --- | --- |
| simulate_response | cycle_index | Python local assignment | int(parts[1]) | Local value used inside simulate_response. | Source [Python], line 368 |
| simulate_response | object_index | Python local assignment | int(parts[2]) | Local value used inside simulate_response. | Source [Python], line 369 |
| simulate_response | position_name | Python local assignment | parts[3] | Local value used inside simulate_response. | Source [Python], line 370 |
| emit_status | line | Python local assignment | f"STATUS;" f"STATE={state['robot_state']};" f"INIT={state['init' | Local value used inside emit_status. | Source [Python], line 421 |
| emit_config | line | Python local assignment | f"CONFIG;" f"CYCLE={state['cycle_index']};" f"OBJECT={state['obj | Local value used inside emit_config. | Source [Python], line 444 |
| parse_semicolon_line | parts | Python local assignment | line.split(";") | Local value used inside parse_semicolon_line. | Source [Python], line 462 |
| parse_semicolon_line | msg_type | Python local assignment | parts[0] | Local value used inside parse_semicolon_line. | Source [Python], line 463 |
| parse_semicolon_line | data | Python local assignment | {} | Local value used inside parse_semicolon_line. | Source [Python], line 465 |
| parse_semicolon_line | k | Python local assignment | p.split("=", 1) | Local value used inside parse_semicolon_line. | Source [Python], line 468 |
| parse_semicolon_line | v | Python local assignment | p.split("=", 1) | Local value used inside parse_semicolon_line. | Source [Python], line 468 |
| handle_arduino_line | msg_type | Python local assignment | parse_semicolon_line(line) | Local value used inside handle_arduino_line. | Source [Python], line 480 |
| handle_arduino_line | data | Python local assignment | parse_semicolon_line(line) | Local value used inside handle_arduino_line. | Source [Python], line 480 |
| handle_arduino_line | ok_text | Python local assignment | line.split(";", 1)[1] if ";" in line else "" | Local value used inside handle_arduino_line. | Source [Python], line 520 |
| handle_arduino_line | position_index | Python local assignment | int(data.get("POSITION", "0")) | Local value used inside handle_arduino_line. | Source [Python], line 523 |
| handle_arduino_line | cycle_index | Python local assignment | int(data.get("CYCLE", state["cycle_index"])) | Local value used inside handle_arduino_line. | Source [Python], line 524 |
| handle_arduino_line | object_index | Python local assignment | int(data.get("OBJECT", state["object_index"])) | Local value used inside handle_arduino_line. | Source [Python], line 525 |
| handle_arduino_line | position_index | Python local assignment | int(data.get("POSITION", "0")) | Local value used inside handle_arduino_line. | Source [Python], line 541 |
| handle_arduino_line | cycle_index | Python local assignment | int(data.get("CYCLE", state["cycle_index"])) | Local value used inside handle_arduino_line. | Source [Python], line 542 |
| handle_arduino_line | object_index | Python local assignment | int(data.get("OBJECT", state["object_index"])) | Local value used inside handle_arduino_line. | Source [Python], line 543 |
| handle_arduino_line | name | Python local assignment | data.get("NAME", "?") | Local value used inside handle_arduino_line. | Source [Python], line 563 |
| handle_arduino_line | status | Python local assignment | data.get("STATUS", "RECEIVED") | Local value used inside handle_arduino_line. | Source [Python], line 564 |
| handle_arduino_line | message | Python local assignment | f"Bouton physique {name} : {status}" | Local value used inside handle_arduino_line. | Source [Python], line 565 |
| apply_config | cycle_index | Python local assignment | int(config_cycle.get()) - 1 | Local value used inside apply_config. | Source [Python], line 579 |
| apply_config | object_index | Python local assignment | objects.index(config_object.get()) | Local value used inside apply_config. | Source [Python], line 580 |
| apply_config | mode_index | Python local assignment | modes.index(config_mode.get()) | Local value used inside apply_config. | Source [Python], line 581 |
| apply_config | pieces_value | Python local assignment | int(config_pieces.get()) | Local value used inside apply_config. | Source [Python], line 582 |
| apply_config | speed_value | Python local assignment | float(config_speed.get()) | Local value used inside apply_config. | Source [Python], line 583 |
| set_servo | value | Python local assignment | float(value) | Local value used inside set_servo. | Source [Python], line 673 |
| set_servo | value | Python local assignment | max(0.0, min(180.0, value)) | Local value used inside set_servo. | Source [Python], line 674 |
| update_position_command_pre view | cycle_index | Python local assignment | int(position_cycle_var.get()) - 1 | Local value used inside update_position_command_preview. | Source [Python], line 687 |
| update_position_command_pre view | object_index | Python local assignment | objects.index(position_object_var.ge t()) | Local value used inside update_position_command_preview. | Source [Python], line 688 |
| update_position_command_pre view | position_name | Python local assignment | position_name_var.get() | Local value used inside update_position_command_preview. | Source [Python], line 689 |
| update_position_command_pre view | position_index | Python local assignment | position_names.index(position_nam e) | Local value used inside update_position_command_preview. | Source [Python], line 690 |
| update_position_command_pre view | code_name | Python local assignment | position_code_names[position_inde x] | Local value used inside update_position_command_preview. | Source [Python], line 693 |
| Function | Variable | Type | Value | Meaning | Source |
| go_to_selected_position | cycle_index | Python local assignment | int(position_cycle_var.get()) - 1 | Local value used inside go_to_selected_position. | Source [Python], line 703 |
| go_to_selected_position | object_index | Python local assignment | objects.index(position_object_var.ge t()) | Local value used inside go_to_selected_position. | Source [Python], line 704 |
| go_to_selected_position | position_name | Python local assignment | position_name_var.get() | Local value used inside go_to_selected_position. | Source [Python], line 705 |
| go_to_selected_position | position_index | Python local assignment | position_names.index(position_nam e) | Local value used inside go_to_selected_position. | Source [Python], line 706 |
| go_to_selected_position | code_name | Python local assignment | position_code_names[position_inde x] | Local value used inside go_to_selected_position. | Source [Python], line 709 |
| on_tab_changed | selected_tab | Python local assignment | notebook.index(notebook.select()) | Local value used inside on_tab_changed. | Source [Python], line 738 |
| add_servo_control | var | Python local assignment | tk.DoubleVar(value=90.0) | Local value used inside add_servo_control. | Source [Python], line 1037 |
| add_servo_control | scale | Python local assignment | tk.Scale( servo_box, from_=0, to=180, resolution | Local value used inside add_servo_control. | Source [Python], line 1041 |
| add_servo_control | step_box | Python local assignment | tk.Frame(servo_box) | Local value used inside add_servo_control. | Source [Python], line 1059 |

# 6. Data Persistence / EEPROM (With Line Numbers)

EEPROM is used by the Arduino source to preserve calibration and configuration after power loss. The persistent block includes validation via a magic value, position offsets for cycle/object/position combinations, object-specific gripper settings, pickup heights, speed factor, and servo calibration offsets.

| Element | Type | Value | Role | Source |
| --- | --- | --- | --- | --- |
| magic | unsigned long |  | Global value in EEPROM SETTINGS. | Source [Arduino], line 256 |
| savedSpeedFactor | float |  | Global value in EEPROM SETTINGS. | Source [Arduino], line 259 |
| savedServoOffsetA | float |  | Global value in EEPROM SETTINGS. | Source [Arduino], line 260 |
| savedServoOffsetB | float |  | Global value in EEPROM SETTINGS. | Source [Arduino], line 261 |
| savedServoOffsetZ | float |  | Global value in EEPROM SETTINGS. | Source [Arduino], line 262 |
| EEPROM_MAGIC | const unsigned long | 20260427 | Global value in EEPROM SETTINGS. | Source [Arduino], line 265 |
| EEPROM_ADDRESS | const int | 0 | Global value in EEPROM SETTINGS. | Source [Arduino], line 266 |
| eepromData | EEPROMData |  | Global value in EEPROM SETTINGS. | Source [Arduino], line 268 |
| offsetStep | float | 1.0 | Global value in EEPROM SETTINGS. | Source [Arduino], line 274 |
| gripStep | float | 0.05 | Global value in EEPROM SETTINGS. | Source [Arduino], line 275 |
| speedStep | float | 0.1 | Global value in EEPROM SETTINGS. | Source [Arduino], line 276 |
| servoOffsetStep | float | 1.0 | Global value in EEPROM SETTINGS. | Source [Arduino], line 277 |
| PositionOffset.dx | float |  | Field of PositionOffset. | Source [Arduino], line 245 |
| PositionOffset.dy | float |  | Field of PositionOffset. | Source [Arduino], line 246 |
| PositionOffset.dz | float |  | Field of PositionOffset. | Source [Arduino], line 247 |
| GripSettings.openSetpoint | float |  | Field of GripSettings. | Source [Arduino], line 251 |
| GripSettings.closeSetpoint | float |  | Field of GripSettings. | Source [Arduino], line 252 |
| EEPROMData.magic | unsigned long |  | Field of EEPROMData. | Source [Arduino], line 256 |
| EEPROMData.offsets | PositionOffset[NB_CYCLES][NB_O BJECTS][NB_POSITIONS] |  | Field of EEPROMData. | Source [Arduino], line 257 |
| EEPROMData.grip | GripSettings[NB_OBJECTS] |  | Field of EEPROMData. | Source [Arduino], line 258 |
| EEPROMData.savedSpeedFactor | float |  | Field of EEPROMData. | Source [Arduino], line 259 |
| EEPROMData.savedServoOffsetA | float |  | Field of EEPROMData. | Source [Arduino], line 260 |
| EEPROMData.savedServoOffsetB | float |  | Field of EEPROMData. | Source [Arduino], line 261 |
| EEPROMData.savedServoOffsetZ | float |  | Field of EEPROMData. | Source [Arduino], line 262 |
| Element | Type | Value | Role | Source |
| setDefaultEEPROMData | function |  | Manages persistent calibration/configuration or position offsets. | Source [Arduino], line 324 |
| applySavedSettingsToRobot | function |  | Manages persistent calibration/configuration or position offsets. | Source [Arduino], line 352 |
| saveSettingsToEEPROM | function |  | Manages persistent calibration/configuration or position offsets. | Source [Arduino], line 375 |
| loadSettingsFromEEPROM | function |  | Manages persistent calibration/configuration or position offsets. | Source [Arduino], line 386 |
| resetEEPROMToDefaults | function |  | Manages persistent calibration/configuration or position offsets. | Source [Arduino], line 397 |
| moveToEEPROMPositionForObject | function |  | Manages persistent calibration/configuration or position offsets. | Source [Arduino], line 2180 |
| moveToEEPROMPosition | function |  | Manages persistent calibration/configuration or position offsets. | Source [Arduino], line 2219 |

Design implication: saving one structured block is simpler than maintaining many unrelated EEPROM offsets. The magic value protects against stale or incompatible data after firmware changes.

# 7. Function Table (Complete, With Line Numbers)

## 7.1 Arduino functions

| Function | Return type | Arguments | Section | Line | Description |
| --- | --- | --- | --- | --- | --- |
| constrainOffset | float | float value | EEPROM SETTINGS | Source [Arduino], line 279 | Manages persistent calibration/configuration or position offsets. |
| getObjectNameByIndex | const char* | int obj | EEPROM SETTINGS | Source [Arduino], line 283 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| getCycleNameByIndex | const char* | int cyc | EEPROM SETTINGS | Source [Arduino], line 289 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| applyObjectGeometry | void | int objectIndex | EEPROM SETTINGS | Source [Arduino], line 294 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| setDefaultEEPROMData | void |  | EEPROM SETTINGS | Source [Arduino], line 324 | Manages persistent calibration/configuration or position offsets. |
| applySavedSettingsToRobot | void |  | General | Source [Arduino], line 352 | Manages persistent calibration/configuration or position offsets. |
| applyGripperSettingsForObject | void | int objectIndex | General | Source [Arduino], line 364 | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. |
| applyFullConfiguration | void |  | General | Source [Arduino], line 369 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| saveSettingsToEEPROM | void |  | General | Source [Arduino], line 375 | Manages persistent calibration/configuration or position offsets. |
| loadSettingsFromEEPROM | void |  | General | Source [Arduino], line 386 | Manages persistent calibration/configuration or position offsets. |
| resetEEPROMToDefaults | void |  | General | Source [Arduino], line 397 | Manages persistent calibration/configuration or position offsets. |
| getOffset | PositionOffset | int cycle, int object, int position | General | Source [Arduino], line 405 | Manages persistent calibration/configuration or position offsets. |
| applyMachinePieceCentering | void | Point3D &target, int position | and the real piece center. | Source [Arduino], line 416 | Function in and the real piece center.. |
| getCorrectedPosition | Point3D | int cycle, int object, int position | and the real piece center. | Source [Arduino], line 435 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| getCorrectedPrehensionPositio n | Point3D | int cycle, int object, int position | and the real piece center. | Source [Arduino], line 447 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| pauseISR | void |  | INTERRUPTS | Source [Arduino], line 536 | Function in INTERRUPTS. |
| timerISR | void |  | INTERRUPTS | Source [Arduino], line 542 | Function in INTERRUPTS. |
| getButtonIndex | int | int pin | BUTTON LOGIC | Source [Arduino], line 549 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| buttonPressed | bool | int pin | BUTTON LOGIC | Source [Arduino], line 556 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| getCycleText | const char* |  | TEXT HELPERS | Source [Arduino], line 577 | Selects, applies, simulates, or executes cycle/object/mode configuration. |

| Function | Return type | Arguments | Section | Line | Description |
| --- | --- | --- | --- | --- | --- |
| getObjectText | const char* |  | TEXT HELPERS | Source [Arduino], line 581 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| getModeText | const char* |  | TEXT HELPERS | Source [Arduino], line 585 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| getCombineValue | int |  | TEXT HELPERS | Source [Arduino], line 591 | Function in TEXT HELPERS. |
| getRunStateText | const char* |  | TEXT HELPERS | Source [Arduino], line 595 | Function in TEXT HELPERS. |
| drawMenu | void | const char* title, const char* items[], int itemCount, int selected | OLED | Source [Arduino], line 606 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| drawStatusScreen | void |  | OLED | Source [Arduino], line 631 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| drawInfoMessage | void | const char* title, const char* line1, const char* line2 = "" | OLED | Source [Arduino], line 653 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| drawReadyScreen | void |  | OLED | Source [Arduino], line 672 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| refreshScreen | void |  | OLED | Source [Arduino], line 695 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| refreshStatusBetweenMoves | void |  | General | Source [Arduino], line 717 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| updateLEDs | void |  | LEDS | Source [Arduino], line 727 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| configChanged | void |  | LEDS | Source [Arduino], line 744 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| terminalConfigChanged | void |  | LEDS | Source [Arduino], line 761 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| getMenuCount | int | ScreenState s | HMI | Source [Arduino], line 775 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| handleEncoder | void |  | HMI | Source [Arduino], line 786 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| executeCurrentScreen | void |  | HMI | Source [Arduino], line 818 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| applyGripperOpening | void | double pwm | GRIPPER PID FUNCTIONS | Source [Arduino], line 911 | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. |
| applyGripperClosing | void | double pwm | GRIPPER PID FUNCTIONS | Source [Arduino], line 917 | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. |
| stopGripper | void |  | GRIPPER PID FUNCTIONS | Source [Arduino], line 923 | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. |
| openGripper | void |  | GRIPPER PID FUNCTIONS | Source [Arduino], line 934 | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. |
| closeGripper | void |  | GRIPPER PID FUNCTIONS | Source [Arduino], line 946 | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. |
| updateGripperPID | void |  | GRIPPER PID FUNCTIONS | Source [Arduino], line 958 | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. |
| handlePauseInterrupt | void |  | SYSTEM YIELD | Source [Arduino], line 999 | Function in SYSTEM YIELD. |
| systemYield | void |  | SYSTEM YIELD | Source [Arduino], line 1042 | Function in SYSTEM YIELD. |
| smartWait | void | unsigned long ms | SYSTEM YIELD | Source [Arduino], line 1075 | Function in SYSTEM YIELD. |
| degToUs | int | float deg | KINEMATICS | Source [Arduino], line 1087 | Function in KINEMATICS. |
| writeServosMicroseconds | void | float cmdA, float cmdB, float cmdZ | KINEMATICS | Source [Arduino], line 1092 | Controls, tests, synchronizes, or reports servo positions. |
| limitStep | float | float currentValue, float targetValue, float maxStep | KINEMATICS | Source [Arduino], line 1098 | Function in KINEMATICS. |

| Function | Return type | Arguments | Section | Line | Description |
| --- | --- | --- | --- | --- | --- |
| computeIK | JointCmd | float x, float y, float z | KINEMATICS | Source [Arduino], line 1107 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| moveJointsSmoothTo | void | JointCmd target, unsigned long dtMs = 5 | MOVEMENT | Source [Arduino], line 1150 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| trapezoidalProfile | float | float u, float alpha = 0.2 | MOVEMENT | Source [Arduino], line 1172 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| moveLinearSegment | void | Point3D p0, Point3D p1, unsigned long dureeMs, unsigned long dtMs = 20 | MOVEMENT | Source [Arduino], line 1195 | Handles serial protocol parsing, command emission, or status/message generation. |
| moveRectangular | void | float xf, float yf, float zf, float liftHeight, unsigned long tUp, unsigned long tXY, unsigned long tDown | MOVEMENT | Source [Arduino], line 1237 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| moveToReposPosition | void |  | General | Source [Arduino], line 1251 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| returnToReposFromCurrentSer voAngles | bool |  | General | Source [Arduino], line 1255 | Controls, tests, synchronizes, or reports servo positions. |
| initializeFromCartesianPosition | bool | float x, float y, float z | General | Source [Arduino], line 1276 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| performInitialization | void |  | INITIALIZATION ACTION | Source [Arduino], line 1298 | Performs connection/initialization/ready-state transition logic. |
| printCurrentConfiguration | void |  | MAINTENANCE PRINT FUNCTIONS | Source [Arduino], line 1324 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| printMaintenanceNotice | void |  | MAINTENANCE PRINT FUNCTIONS | Source [Arduino], line 1347 | Function in MAINTENANCE PRINT FUNCTIONS. |
| printMainMaintenanceMenu | void |  | MAINTENANCE PRINT FUNCTIONS | Source [Arduino], line 1360 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| printSelectedMaintenanceTarg et | void |  | MAINTENANCE PRINT FUNCTIONS | Source [Arduino], line 1403 | Function in MAINTENANCE PRINT FUNCTIONS. |
| printPositionTable | void | int cycle, int object | General | Source [Arduino], line 1432 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| printCycleTableMenu | void | int cycle | General | Source [Arduino], line 1484 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| updateSelectedOffset | void | char axis, float delta | MAINTENANCE EDIT FUNCTIONS | Source [Arduino], line 1528 | Manages persistent calibration/configuration or position offsets. |
| updateServoOffset | void | char servo, float delta | MAINTENANCE EDIT FUNCTIONS | Source [Arduino], line 1538 | Manages persistent calibration/configuration or position offsets. |
| updateGripSetpoint | void | bool closeSetpoint, float delta | MAINTENANCE EDIT FUNCTIONS | Source [Arduino], line 1561 | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. |
| testLEDsMaintenance | void |  | MAINTENANCE EDIT FUNCTIONS | Source [Arduino], line 1599 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| updateManualServoFromEncod er | void |  | MAINTENANCE MODE | Source [Arduino], line 1633 | Controls, tests, synchronizes, or reports servo positions. |
| readSerialCommand | String |  | MAINTENANCE MODE | Source [Arduino], line 1675 | Handles serial protocol parsing, command emission, or status/message generation. |
| processMainMaintenanceCom mand | void | String cmd | MAINTENANCE MODE | Source [Arduino], line 1686 | Handles serial protocol parsing, command emission, or status/message generation. |
| processTableMaintenanceCom mand | void | String cmd | General | Source [Arduino], line 1797 | Handles serial protocol parsing, command emission, or status/message generation. |
| maintenanceMode | void |  | General | Source [Arduino], line 1835 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| ihmInitializeAction | void |  | This keeps INIT / START / PAUSE exactly the same. | Source [Arduino], line 1924 | Performs connection/initialization/ready-state transition logic. |
| ihmStartAction | void |  | accidentally become the first real arming step. | Source [Arduino], line 1996 | Function in accidentally become the first real arming step.. |
| ihmPauseToggleAction | void |  | General | Source [Arduino], line 2052 | Function in General. |

| Function | Return type | Arguments | Section | Line | Description |
| --- | --- | --- | --- | --- | --- |
| sendButtonTestEvent | void | const char* buttonName | General | Source [Arduino], line 2067 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| setLedByName | void | String ledName, bool on | General | Source [Arduino], line 2074 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| pulseLedByName | void | String ledName | General | Source [Arduino], line 2103 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| handleButtons | void |  | HANDLE BUTTONS | Source [Arduino], line 2139 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| getCycleTimes | void | unsigned long &tUp, unsigned long &tXY, unsigned long &tDown | SPEED HELPER | Source [Arduino], line 2162 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| moveToEEPROMPositionForO bject | void | int cycle, int objectIndex, int position, float liftHeight, unsigned long tUp, unsigned long tXY, unsigned long tDown | SPEED HELPER | Source [Arduino], line 2180 | Manages persistent calibration/configuration or position offsets. |
| moveToPrehensionPositionFor Object | void | int cycle, int objectIndex, int position, float liftHeight, unsigned long tUp, unsigned long tXY, unsigned long tDown | SPEED HELPER | Source [Arduino], line 2185 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| moveToEEPROMPosition | void | int cycle, int position, float liftHeight, unsigned long tUp, unsigned long tXY, unsigned long tDown | Kept for compatibility with old maintenance/test code. | Source [Arduino], line 2219 | Manages persistent calibration/configuration or position offsets. |
| prepareObjectCycle | void | int objectIndex | Kept for compatibility with old maintenance/test code. | Source [Arduino], line 2223 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| SequenceCycle1ForObject | void | int objectIndex | The object is passed in so offsets + gripper setpoints are object-spec | Source [Arduino], line 2235 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| SequenceCycle2ForObject | void | int objectIndex | The object is passed in so offsets + gripper setpoints are object-spec | Source [Arduino], line 2273 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| cycleGomme | void |  | and gripper EEPROM settings. | Source [Arduino], line 2344 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| cycleGobelet | void |  | and gripper EEPROM settings. | Source [Arduino], line 2352 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| cycleCylindre | void |  | and gripper EEPROM settings. | Source [Arduino], line 2360 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| SequenceCycle1 | void |  | Old names kept as wrappers so any older serial/maintenance command sti | Source [Arduino], line 2369 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| SequenceCycle2 | void |  | Old names kept as wrappers so any older serial/maintenance command sti | Source [Arduino], line 2373 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| runSelectedObjectCycle | void |  | Old names kept as wrappers so any older serial/maintenance command sti | Source [Arduino], line 2377 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| updateRobotCycle | void |  | Old names kept as wrappers so any older serial/maintenance command sti | Source [Arduino], line 2387 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| drawTerminalModeScreen | void |  | terminal sends PING or any API command. Cavalier inutile. | Source [Arduino], line 2430 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| enterTerminalMode | void |  | terminal sends PING or any API command. Cavalier inutile. | Source [Arduino], line 2447 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| exitTerminalMode | void |  | terminal sends PING or any API command. Cavalier inutile. | Source [Arduino], line 2454 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| sendStatus | void |  | position: 0=POS_convoyeurEntree, 1=POS_machineA, 2=POS_machineB, 3=POS | Source [Arduino], line 2490 | Function in position: 0=POS_convoyeurEntree, 1=POS_machineA, 2=POS_machineB, 3=POS. |
| sendConfig | void |  | position: 0=POS_convoyeurEntree, 1=POS_machineA, 2=POS_machineB, 3=POS | Source [Arduino], line 2511 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| Function | Return type | Arguments | Section | Line | Description |
| getArg | String | String cmd | position: 0=POS_convoyeurEntree, 1=POS_machineA, 2=POS_machineB, 3=POS | Source [Arduino], line 2520 | Function in position: 0=POS_convoyeurEntree, 1=POS_machineA, 2=POS_machineB, 3=POS. |
| apiPauseToggle | void |  | position: 0=POS_convoyeurEntree, 1=POS_machineA, 2=POS_machineB, 3=POS | Source [Arduino], line 2528 | Handles serial protocol parsing, command emission, or status/message generation. |
| apiGotoPosition | void | String cmd | position: 0=POS_convoyeurEntree, 1=POS_machineA, 2=POS_machineB, 3=POS | Source [Arduino], line 2574 | Handles serial protocol parsing, command emission, or status/message generation. |
| handleApiCommand | void | String cmd | General | Source [Arduino], line 2644 | Handles serial protocol parsing, command emission, or status/message generation. |
| handleApiSerial | void |  | General | Source [Arduino], line 2929 | Handles serial protocol parsing, command emission, or status/message generation. |
| setup | void |  | SETUP & LOOP | Source [Arduino], line 2951 | Arduino lifecycle function for hardware setup or continuous cooperative servicing. |
| loop | void |  | SETUP & LOOP | Source [Arduino], line 3018 | Arduino lifecycle function for hardware setup or continuous cooperative servicing. |

## 7.2 Python functions

| Function | Arguments | Section | Line | Description |
| --- | --- | --- | --- | --- |
| find_arduino_port |  | SERIAL HELPERS | Source [Python], line 93 | Function in SERIAL HELPERS. |
| resolve_serial_port |  | SERIAL HELPERS | Source [Python], line 111 | Handles serial protocol parsing, command emission, or status/message generation. |
| connect_arduino |  | SERIAL HELPERS | Source [Python], line 118 | Performs connection/initialization/ready-state transition logic. |
| disconnect_arduino |  | The Arduino will lock its physical OLED to "MAINTENANCE TERMINAL". | Source [Python], line 170 | Performs connection/initialization/ready-state transition logic. |
| reconnect_arduino |  | The Arduino will lock its physical OLED to "MAINTENANCE TERMINAL". | Source [Python], line 187 | Performs connection/initialization/ready-state transition logic. |
| send_command | cmd | The Arduino will lock its physical OLED to "MAINTENANCE TERMINAL". | Source [Python], line 192 | Handles serial protocol parsing, command emission, or status/message generation. |
| read_arduino |  | The Arduino will lock its physical OLED to "MAINTENANCE TERMINAL". | Source [Python], line 210 | Function in The Arduino will lock its physical OLED to "MAINTENANCE TERMINAL".. |
| simulate_response | cmd | SIMULATION MODE | Source [Python], line 226 | Function in SIMULATION MODE. |
| finish_sim_init |  | General | Source [Python], line 386 | Performs connection/initialization/ready-state transition logic. |
| run_sim_cycle |  | General | Source [Python], line 394 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| loop |  | General | Source [Python], line 395 | Arduino lifecycle function for hardware setup or continuous cooperative servicing. |
| emit_status |  | General | Source [Python], line 420 | Handles serial protocol parsing, command emission, or status/message generation. |
| emit_config |  | General | Source [Python], line 443 | Handles serial protocol parsing, command emission, or status/message generation. |
| parse_semicolon_line | line | PARSER | Source [Python], line 461 | Handles serial protocol parsing, command emission, or status/message generation. |
| handle_arduino_line | line | PARSER | Source [Python], line 474 | Handles serial protocol parsing, command emission, or status/message generation. |
| apply_config |  | GUI ACTIONS | Source [Python], line 578 | Selects, applies, simulates, or executes cycle/object/mode configuration. |
| ihm_init |  | GUI ACTIONS | Source [Python], line 604 | Performs connection/initialization/ready-state transition logic. |
| ihm_play |  | GUI ACTIONS | Source [Python], line 612 | Function in GUI ACTIONS. |
| ihm_pause |  | Use APPLY CONFIG before INIT if you changed cycle/object/mode/pieces/s | Source [Python], line 619 | Function in Use APPLY CONFIG before INIT if you changed cycle/object/mode/pieces/s. |
| test_serial_link |  | Use APPLY CONFIG before INIT if you changed cycle/object/mode/pieces/s | Source [Python], line 624 | Handles serial protocol parsing, command emission, or status/message generation. |
| test_init_led |  | Use APPLY CONFIG before INIT if you changed cycle/object/mode/pieces/s | Source [Python], line 630 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| Function | Arguments | Section | Line | Description |
| start_button_test_mode |  | LED AND BUTTON TEST ACTIONS | Source [Python], line 638 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| stop_button_test_mode |  | LED AND BUTTON TEST ACTIONS | Source [Python], line 647 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| led_set | led_name, value | LED AND BUTTON TEST ACTIONS | Source [Python], line 652 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| led_pulse | led_name | LED AND BUTTON TEST ACTIONS | Source [Python], line 656 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| led_auto |  | LED AND BUTTON TEST ACTIONS | Source [Python], line 660 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| go_repos_from_terminal |  | SERVO TEST ACTIONS | Source [Python], line 667 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| set_servo | servo_name, value | SERVO TEST ACTIONS | Source [Python], line 672 | Controls, tests, synchronizes, or reports servo positions. |
| step_servo | servo_name, delta | SERVO TEST ACTIONS | Source [Python], line 678 | Controls, tests, synchronizes, or reports servo positions. |
| update_position_command_preview |  | POSITION CHECK ACTIONS | Source [Python], line 685 | Handles serial protocol parsing, command emission, or status/message generation. |
| go_to_selected_position |  | POSITION CHECK ACTIONS | Source [Python], line 701 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| go_to_repos_from_position_tab |  | 3 Sortie -> POS_convoyeurSortie | Source [Python], line 733 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| on_tab_changed | event | 3 Sortie -> POS_convoyeurSortie | Source [Python], line 737 | Function in 3 Sortie -> POS_convoyeurSortie. |
| sync_servo_sliders_from_state |  | GUI DISPLAY | Source [Python], line 758 | Controls, tests, synchronizes, or reports servo positions. |
| update_dashboard |  | Sliders may not exist during startup. | Source [Python], line 769 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| draw_oled |  | Sliders may not exist during startup. | Source [Python], line 800 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| oled_text | x, y, text | but this preview should show the live robot state from Arduino STATUS | Source [Python], line 819 | Updates display/UI state, menu screens, dashboard fields, or terminal preview. |
| log | message | but this preview should show the live robot state from Arduino STATUS | Source [Python], line 830 | Function in but this preview should show the live robot state from Arduino STATUS . |
| add_servo_control | row, servo_name, label | TAB 2 - SINGLE SERVO TESTS | Source [Python], line 1036 | Controls, tests, synchronizes, or reports servo positions. |
| add_led_control | row, led_name, label | TAB 3 - BUTTON AND LED TESTS | Source [Python], line 1113 | Processes physical IHM input/output, button events, LED tests, or encoder interaction. |
| add_position_quick_button | position_name, row, column | TAB 4 - POSITION CHECK | Source [Python], line 1223 | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. |
| on_close |  | STARTUP | Source [Python], line 1307 | Function in STARTUP. |

# 8. Core Logic Explanation (With References)

8.1 Initialization flow applyFullConfiguration contributes to embedded initialization: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 369). loadSettingsFromEEPROM contributes to embedded initialization: Manages persistent calibration/configuration or position offsets. (Source [Arduino], line 386).

moveToReposPosition contributes to embedded initialization: Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. (Source [Arduino], line 1251).

initializeFromCartesianPosition contributes to embedded initialization: Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. (Source [Arduino], line 1276). performInitialization contributes to embedded initialization: Performs connection/initialization/ready-state transition logic. (Source [Arduino], line 1298). setup contributes to embedded initialization: Arduino lifecycle function for hardware setup or continuous cooperative servicing. (Source [Arduino], line 2951).

connect_arduino contributes to terminal-side initialization/configuration: Performs connection/initialization/ready-state transition logic. (Source [Python], line 118). apply_config contributes to terminal-side initialization/configuration: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Python], line 578).

ihm_init contributes to terminal-side initialization/configuration: Performs connection/initialization/ready-state transition logic. (Source [Python], line 604).

## 8.2 Main execution and state flow

getCycleNameByIndex supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 289). getCycleText supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 577). handlePauseInterrupt supports runtime execution/state transitions: Function in SYSTEM YIELD. (Source [Arduino], line 999). systemYield supports runtime execution/state transitions: Function in SYSTEM YIELD. (Source [Arduino], line 1042). smartWait supports runtime execution/state transitions: Function in SYSTEM YIELD. (Source [Arduino], line 1075). printCycleTableMenu supports runtime execution/state transitions: Updates display/UI state, menu screens, dashboard fields, or terminal preview. (Source [Arduino], line 1484). getCycleTimes supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2162). prepareObjectCycle supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2223).

SequenceCycle1ForObject supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2235). SequenceCycle2ForObject supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2273). cycleGomme supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2344). cycleGobelet supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2352). cycleCylindre supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2360).

SequenceCycle1 supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2369). SequenceCycle2 supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2373).

runSelectedObjectCycle supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2377). updateRobotCycle supports runtime execution/state transitions: Selects, applies, simulates, or executes cycle/object/mode configuration. (Source [Arduino], line 2387).

loop supports runtime execution/state transitions: Arduino lifecycle function for hardware setup or continuous cooperative servicing. (Source [Arduino], line 3018).

## 8.3 Python event and parser flow

send_command is part of the terminal event chain: Handles serial protocol parsing, command emission, or status/message generation. (Source [Python], line 192). read_arduino is part of the terminal event chain: Function in The Arduino will lock its physical OLED to "MAINTENANCE TERMINAL".. (Source [Python], line 210). parse_semicolon_line is part of the terminal event chain: Handles serial protocol parsing, command emission, or status/message generation. (Source [Python], line 461). handle_arduino_line is part of the terminal event chain: Handles serial protocol parsing, command emission, or status/message generation. (Source [Python], line 474). update_dashboard is part of the terminal event chain: Updates display/UI state, menu screens, dashboard fields, or terminal preview. (Source [Python], line 769). draw_oled is part of the terminal event chain: Updates display/UI state, menu screens, dashboard fields, or terminal preview. (Source [Python], line 800). on_tab_changed is part of the terminal event chain: Function in 3 Sortie -> POS_convoyeurSortie. (Source [Python], line 737).

# 9. Algorithms & Computation

| Algorithm / Function | Section | Computation / transformation | Source |
| --- | --- | --- | --- |
| applyObjectGeometry | EEPROM SETTINGS | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 294 |
| applyMachinePieceCentering | and the real piece center. | Function in and the real piece center.. | Source [Arduino], line 416 |
| getCorrectedPosition | and the real piece center. | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 435 |
| getCorrectedPrehensionPosition | and the real piece center. | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 447 |
| degToUs | KINEMATICS | Function in KINEMATICS. | Source [Arduino], line 1087 |
| limitStep | KINEMATICS | Function in KINEMATICS. | Source [Arduino], line 1098 |
| computeIK | KINEMATICS | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 1107 |
| trapezoidalProfile | MOVEMENT | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 1172 |
| moveLinearSegment | MOVEMENT | Handles serial protocol parsing, command emission, or status/message generation. | Source [Arduino], line 1195 |
| moveRectangular | MOVEMENT | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 1237 |
| simulate_response | SIMULATION MODE | Function in SIMULATION MODE. | Source [Python], line 226 |
| emit_status | General | Handles serial protocol parsing, command emission, or status/message generation. | Source [Python], line 420 |
| emit_config | General | Handles serial protocol parsing, command emission, or status/message generation. | Source [Python], line 443 |
| parse_semicolon_line | PARSER | Handles serial protocol parsing, command emission, or status/message generation. | Source [Python], line 461 |
| update_position_command_preview | POSITION CHECK ACTIONS | Handles serial protocol parsing, command emission, or status/message generation. | Source [Python], line 685 |
| sync_servo_sliders_from_state | GUI DISPLAY | Controls, tests, synchronizes, or reports servo positions. | Source [Python], line 758 |
| update_dashboard | Sliders may not exist during startup. | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Python], line 769 |

The most important embedded computation is inverse kinematics: Cartesian targets are converted into servo commands. The motion layer then smooths these commands with step limiting and trajectory interpolation. Python computation is primarily data transformation: parsing semicolon messages, updating the state dictionary, and refreshing Tkinter variables.

# 10. Control Systems (PID or Equivalent)

General PID form used for explanation: error = setpoint - measured; output = Kp x error + Ki x integral(error) + Kd x derivative(error). In this system, the gripper setpoint is the target feedback/current/tension value, the measurement comes from analog feedback pins, and the output is motor PWM/direction. Safety limits stop the gripper when object/contact or open-limit conditions are reached.

| Scope | Element | Type | Value | Role | Source |
| --- | --- | --- | --- | --- | --- |
| global | PIN_GRIP_PWM | const int | 3 | Global value in PIN MAPPING. | Source [Arduino], line 18 |
| global | CURRENT_CLOSE_PIN | const int | A4 | Global value in PIN MAPPING. | Source [Arduino], line 20 |
| global | CURRENT_OPEN_PIN | const int | A5 | Global value in PIN MAPPING. | Source [Arduino], line 21 |
| global | currentX | float | 0.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 89 |
| global | currentY | float | 130.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 90 |
| global | currentZ | float | 120.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 91 |
| global | currentCmdA | float | 90.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 93 |
| global | currentCmdB | float | 90.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 94 |
| global | currentCmdZ | float | 90.0 | Global value in ROBOT KINEMATICS. | Source [Arduino], line 95 |
| global | currentCycleType | int | 0 | Global value in HMI GLOBAL STATES. | Source [Arduino], line 201 |
| global | currentObject | int | 0 | Global value in HMI GLOBAL STATES. | Source [Arduino], line 202 |
| global | currentMode | int | 1 | Global value in HMI GLOBAL STATES. | Source [Arduino], line 203 |
| Scope | Element | Type | Value | Role | Source |
| global | currentCombine | int | 0 | Global value in HMI GLOBAL STATES. | Source [Arduino], line 204 |
| global | currentPiece | int | 0 | Global value in HMI GLOBAL STATES. | Source [Arduino], line 205 |
| global | consigneOuverture | double | 1.80 | Global value in IMPORTANT: declared here before EEPROM functions use them. | Source [Arduino], line 235 |
| global | consigneFermeture | double | 1.40 | Global value in IMPORTANT: declared here before EEPROM functions use them. | Source [Arduino], line 236 |
| global | seuilOuvertureMax | double | 2.30 | Global value in IMPORTANT: declared here before EEPROM functions use them. | Source [Arduino], line 238 |
| global | seuilFermetureMax | double | 2.60 | Global value in IMPORTANT: declared here before EEPROM functions use them. | Source [Arduino], line 239 |
| global | gripperState | GripperState | GRIPPER_STOPPED | Global value in GRIPPER PID. | Source [Arduino], line 487 |
| global | gripperPWM_PID | double | 0.0 | Global value in GRIPPER PID. | Source [Arduino], line 489 |
| global | measuredTension | double | 0.0 | Global value in GRIPPER PID. | Source [Arduino], line 490 |
| global | gripperSetpoint | double | 0.0 | Global value in GRIPPER PID. | Source [Arduino], line 491 |
| global | filteredTension | double | 0.0 | Global value in GRIPPER PID. | Source [Arduino], line 493 |
| global | previousGripPWM | double | 0.0 | Global value in GRIPPER PID. | Source [Arduino], line 496 |
| global | maxGripPWMVariation | const double | 15.0 | Global value in GRIPPER PID. | Source [Arduino], line 497 |
| global | gripperDone | bool | false | Global value in GRIPPER PID. | Source [Arduino], line 499 |
| global | objectDetected | bool | false | Global value in GRIPPER PID. | Source [Arduino], line 500 |
| global | openLimitDetected | bool | false | Global value in GRIPPER PID. | Source [Arduino], line 501 |
| updateGripperPID | rawTension | double |  | Local variable used inside updateGripperPID. | Source [Arduino], line 964 |
| function | applyGripperSettingsForObject | void |  | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. | Source [Arduino], line 364 |
| function | applyGripperOpening | void |  | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. | Source [Arduino], line 911 |
| function | applyGripperClosing | void |  | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. | Source [Arduino], line 917 |
| function | stopGripper | void |  | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. | Source [Arduino], line 923 |
| function | openGripper | void |  | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. | Source [Arduino], line 934 |
| function | closeGripper | void |  | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. | Source [Arduino], line 946 |
| function | updateGripperPID | void |  | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. | Source [Arduino], line 958 |

# 11. Communication / Interface Protocol

The serial protocol is intentionally human-readable. Python sends uppercase commands with optional arguments followed by a newline. Arduino and Python exchange semicolon-separated messages such as STATUS;KEY=VALUE;KEY=VALUE. The Python parser splits the line, converts key/value pairs into a dictionary, and refreshes the dashboard.

| Command / message | Direction | Meaning | Source |
| --- | --- | --- | --- |
| BUTTON_TEST_OFF | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2480 |
| BUTTON_TEST_OFF | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 649 |
| BUTTON_TEST_ON | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2480 |
| BUTTON_TEST_ON | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 644 |
| CONFIG | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 39 |
| CONFIG | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 445 |
| ERR | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 1259 |
| ERR | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 291 |
| EVENT | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2068 |
| EVENT | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 561 |
| GET_CONFIG | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2468 |
| GET_CONFIG | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 162 |

| Command / message | Direction | Meaning | Source |
| --- | --- | --- | --- |
| GET_STATUS | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2467 |
| GET_STATUS | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 161 |
| GOTO_POSITION | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2483 |
| GOTO_POSITION | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 349 |
| GO_NAMED_POSITION | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 366 |
| GO_REPOS | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2477 |
| GO_REPOS | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 669 |
| GRIPPER_CLOSING | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 484 |
| GRIPPER_OPENING | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 483 |
| GRIPPER_STOPPED | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 482 |
| INITIALIZE | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2474 |
| INITIALIZE | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 278 |
| LED_AUTO | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2875 |
| LED_AUTO | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 661 |
| LED_PULSE | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2482 |
| LED_PULSE | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 657 |
| LED_SET | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2096 |
| LED_SET | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 653 |
| OFFSET | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 1405 |
| OK | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 830 |
| OK | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 148 |
| PAUSE_TOGGLE | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2476 |
| PAUSE_TOGGLE | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 312 |
| PING | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2428 |
| PING | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 143 |
| PONG | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2668 |
| PONG | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 148 |
| RUN_CYCLE | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2475 |
| RUN_CYCLE | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 288 |
| SERVO_SET | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2478 |
| SERVO_SET | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 675 |
| SERVO_STEP | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2479 |
| SERVO_STEP | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 679 |
| SET_CYCLE | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 764 |
| SET_CYCLE | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 250 |
| SET_MODE | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 764 |
| SET_MODE | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 262 |
| SET_OBJECT | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 764 |
| SET_OBJECT | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 256 |
| SET_PIECES | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 764 |
| SET_PIECES | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 268 |
| SET_SPEED | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 764 |
| SET_SPEED | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 273 |
| Command / message | Direction | Meaning | Source |
| STATUS | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2070 |
| STATUS | Arduino to Python / parsed by Python | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 148 |
| TERMINAL_OFF | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2465 |
| TERMINAL_OFF | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 237 |
| TERMINAL_ON | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2464 |
| TERMINAL_ON | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 139 |
| TEST_INIT_LED | Arduino handler / embedded response logic | Text serial protocol command or message used by the terminal/controller interface. | Source [Arduino], line 2884 |
| TEST_INIT_LED | Python to Arduino | Text serial protocol command or message used by the terminal/controller interface. | Source [Python], line 632 |

# 12. Interface / UI Description

| Screen / Tab / Function | Type | Purpose | Source |
| --- | --- | --- | --- |
| Onglet 1 - Tests de cycle | Python Tkinter tab | Displayed tab in the maintenance terminal. | Source [Python], line 930 |
| Onglet 2 - Tests servos | Python Tkinter tab | Displayed tab in the maintenance terminal. | Source [Python], line 1023 |
| Onglet 3 - Tests boutons / LED | Python Tkinter tab | Displayed tab in the maintenance terminal. | Source [Python], line 1083 |
| Onglet 4 - Test positions | Python Tkinter tab | Displayed tab in the maintenance terminal. | Source [Python], line 1144 |
| SCREEN_WIDTH | Arduino OLED screen state | Preprocessor constant or macro used by the embedded controller. | Source [Arduino], line 41 |
| SCREEN_HEIGHT | Arduino OLED screen state | Preprocessor constant or macro used by the embedded controller. | Source [Arduino], line 42 |
| SCREEN_ADDRESS | Arduino OLED screen state | Preprocessor constant or macro used by the embedded controller. | Source [Arduino], line 44 |
| drawMenu | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Arduino], line 606 |
| drawStatusScreen | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Arduino], line 631 |
| drawInfoMessage | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Arduino], line 653 |
| drawReadyScreen | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Arduino], line 672 |
| getMenuCount | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Arduino], line 775 |
| printMainMaintenanceMenu | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Arduino], line 1360 |
| printPositionTable | UI function | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 1432 |
| printCycleTableMenu | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Arduino], line 1484 |
| processTableMaintenanceCommand | UI function | Handles serial protocol parsing, command emission, or status/message generation. | Source [Arduino], line 1797 |
| drawTerminalModeScreen | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Arduino], line 2430 |
| go_to_repos_from_position_tab | UI function | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Python], line 733 |
| on_tab_changed | UI function | Function in 3 Sortie -> POS_convoyeurSortie. | Source [Python], line 737 |
| update_dashboard | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Python], line 769 |
| draw_oled | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Python], line 800 |
| oled_text | UI function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Python], line 819 |

# 13. Execution Flow / Cycles (With Line Numbers)

| Step / element | Type | Purpose | Source |
| --- | --- | --- | --- |
| Point3D basePositions[NB_CYCLES][NB_POSITIONS] = { | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 126 |
| Point3D base = basePositions[cycle][position]; | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 436 |
| Point3D base = basePositions[cycle][position]; | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 450 |
| // The Python UI sends SET_CYCLE / SET_OBJECT / SET_MODE / SET_PIECES / SET_SPEED | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 764 |

| Step / element | Type | Purpose | Source |
| --- | --- | --- | --- |
| Point3D base = basePositions[maintCycle][maintPosition]; | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 1411 |
| Point3D base = basePositions[cycle][p]; | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 1443 |
| Serial.println(F("OK;RUN_CYCLE_START")); | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 2048 |
| // SET_CYCLE 0/1 | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 2469 |
| // RUN_CYCLE | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 2475 |
| else if (upper.startsWith("SET_CYCLE")) { | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 2677 |
| Serial.println(F("OK;SET_CYCLE")); | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 2683 |
| else if (upper == "RUN_CYCLE") { | Cycle/position line | Defines, selects, or triggers cycle and position behavior. | Source [Arduino], line 2916 |
| getObjectNameByIndex | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 283 |
| getCycleNameByIndex | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 289 |
| applyObjectGeometry | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 294 |
| applyGripperSettingsForObject | Arduino function | Controls gripper motor behavior, feedback regulation, PID state, or safety stopping. | Source [Arduino], line 364 |
| getCorrectedPosition | Arduino function | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 435 |
| getCorrectedPrehensionPosition | Arduino function | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 447 |
| getCycleText | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 577 |
| getObjectText | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 581 |
| getModeText | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 585 |
| moveToReposPosition | Arduino function | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 1251 |
| initializeFromCartesianPosition | Arduino function | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 1276 |
| printPositionTable | Arduino function | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 1432 |
| printCycleTableMenu | Arduino function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Arduino], line 1484 |
| maintenanceMode | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 1835 |
| getCycleTimes | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2162 |
| moveToEEPROMPositionForObject | Arduino function | Manages persistent calibration/configuration or position offsets. | Source [Arduino], line 2180 |
| moveToPrehensionPositionForObject | Arduino function | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Arduino], line 2185 |
| moveToEEPROMPosition | Arduino function | Manages persistent calibration/configuration or position offsets. | Source [Arduino], line 2219 |
| prepareObjectCycle | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2223 |
| SequenceCycle1ForObject | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2235 |
| SequenceCycle2ForObject | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2273 |
| cycleGomme | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2344 |
| cycleGobelet | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2352 |
| cycleCylindre | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2360 |
| SequenceCycle1 | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2369 |
| SequenceCycle2 | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2373 |
| runSelectedObjectCycle | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2377 |
| updateRobotCycle | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2387 |
| drawTerminalModeScreen | Arduino function | Updates display/UI state, menu screens, dashboard fields, or terminal preview. | Source [Arduino], line 2430 |
| enterTerminalMode | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2447 |
| exitTerminalMode | Arduino function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Arduino], line 2454 |
| apiGotoPosition | Arduino function | Handles serial protocol parsing, command emission, or status/message generation. | Source [Arduino], line 2574 |
| simulate_response | Python function | Function in SIMULATION MODE. | Source [Python], line 226 |
| run_sim_cycle | Python function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Python], line 394 |
| apply_config | Python function | Selects, applies, simulates, or executes cycle/object/mode configuration. | Source [Python], line 578 |
| Step / element | Type | Purpose | Source |
| ihm_play | Python function | Function in GUI ACTIONS. | Source [Python], line 612 |
| update_position_command_preview | Python function | Handles serial protocol parsing, command emission, or status/message generation. | Source [Python], line 685 |
| go_to_selected_position | Python function | Computes or executes robot movement, positioning, inverse kinematics, or motion smoothing. | Source [Python], line 701 |

Cycle operation follows a traceable order: select configuration, initialize to a known repos state, run the selected cycle if initialized, service pause/resume through cooperative checks, and return/update state through serial status. Position and servo tests are isolated into maintenance tabs so test actions remain separated from normal cycle execution.

# 14. Design Justification

The architecture uses Arduino as the safety-critical hardware authority and Python as the operator/maintenance layer. This is appropriate because the robot must keep deterministic hardware logic close to the pins while still allowing a rich desktop diagnostic interface.

EEPROM persistence is justified because real robot calibration is iterative. Servo offsets, position offsets, gripper setpoints, pickup heights, and speed factor must survive resets without requiring recompilation.

The readable serial protocol is justified for student/engineering validation because commands can be logged and tested manually. The trade-off is synchronization risk: every command name used by Python must match the Arduino handler.

The main limitations are global-state complexity, the need to physically verify EEPROM values on hardware, and the need to keep UI tabs consistent with embedded command support. The OK/ERR/STATUS feedback design reduces this risk by making acceptance/rejection explicit.

# Appendix A - Coverage Summary

Cobot_French.ino: 3024 lines analyzed; detected 156 top-level declarations, 50 local declarations, 8 data type declarations, 19 structure fields, and 105 functions.

Cobot_Terminal_French.py: 1319 lines analyzed; detected 65 top-level assignments/widgets, 59 local assignments, 6 imports, and 42 functions.

All source references use line numbers from the uploaded files at generation time.